| FEATURES: 18 CR2032 lithium battery 210mAh Very low power consumption Active mode Sleep mode Stop mode Flickering Mode Seconds, Minutes, Hours Week Day, Day, Month, & Year 3 Alarms Settings, Sound & lights (optional) Buzzer built in. Auto power OFF (after 15 Seconds) Manual power OFF 138 Colored LED lights Key tone operation. (Optional). Hourly Chime. (Optional). AM/PM indicator. Alarm activated display Green, Yellow, & Red Colors supported SWITCHES: 2 Switch 1: Press to access Screens 1 to 8 Switch 2: Press to Shut OFF display Switch 1& 2: Press & hold simultaneously for Time Setting SCREENS: 8 Screen 1 = (1 press) Hour, Minute, & Seconds Screen 2 = (2 presses) Day of the week Screen 2 = Alarm 1, 2, 3, Hour Chime, & Key tone Screen 3 = (3 presses) Year, Month, Day of the Month Screen 4 = (4 presses) Alarm 1 Screen 5 = (5 presses) Alarm 2 Screen 6 = (6 presses) Alarm 3 Screen 7 = (Hour Chime Setting.) Screen 8 = (Key tone setting) MODES: 4 Active Mode = display ON (clock running) Sleep Mode = display OFF (clock running) Stop Mode = display OFF (clock stop running) Flickering Mode = Flashing lights (Hour, Minute & Second) CODES: 11 Red = Hours & every Fifth Minute & every Fifth Second Yellow = Minutes & Seconds 1 & 3 after each Fifth Green = Minutes & Seconds 2 & 4 after each Fifth Hours 1-12 = Years: e.g. Hour 1 = Year 2001 etc. Hours 12-6 = Days of week e.g. Hour 12 = Saturday Minutes 5,10,15,20,etc, = Months e.g. Minute 5 = January Seconds 1-31 = Days of the Month e.g. second 1 = 1st. Flashing light (Key Tone, Hour Chime) = OFF Stable light (Key Tone, Hour Chime) = ON AM = light OFF PM = light ON IMPORTANT: After 15 Seconds (Active Mode) Displays will be deactivated. 1/ TIME & DATE SETTINGS: Press & hold Switch 1 & Switch 2 simultaneously until Seconds light flashes Press & hold Switch 1 to bring the Seconds to 00 position Press Switch 2 once to set the desired Second Minute’s light will flash Press Switch 1 repeatedly to change the Minute Press Switch 2 once to set the desired Minute Hour’s light will flash Press Switch 1 repeatedly to change the Hour, AM or PM Press Switch 2 once to set the desired Hour Days of the week light will flash. (Hours 12-6) | Press Switch 1 repeatedly to change the Day (Sat – Friday) Press Switch 2 once to set the desired Day Year’s Light will flash. (Each Hour = 1 Year) Press Switch 1 repeatedly to change the Year Press Switch 2 once to set the Year Month’s light will flash. (Fifth of each Minute) Press Switch 1 repeatedly to change the Month Press Switch 2 once to set the Month Days of the Month light will flash. (Second 1 to second 31) Press Switch 1 repeatedly to change the Day Press Switch 2 to set the desired day 2/ ALARM SETTINGS: Setting Alarm 1 Press & hold Switch 1 until Alarm 1 light flashes then release it Press Switch 2 once to activate the Minute Press Switch 1 repeatedly to set the desired Minute Press Switch 2 once to activate the Hour Press Switch 1 repeatedly to set the desired Hour, AM or PM Press Switch 2 to set Alarm 1 ON. Setting Alarm 2 Press & hold Switch 1 until Alarm 1 light flashes then release it Press Switch 1 once to activate Alarm 2. Press Switch 2 once to activate the Minute. Press Switch 1 repeatedly to set the desired Minute Press Switch 2 once to activate the Hour. Press Switch 1 repeatedly to set the desired Hour, AM or PM Press Switch 2 to set Alarm 2 ON. Setting Alarm 3 Press & hold Switch 1 until Alarm 1 light flashes then release it Press Switch 1 once to activate Alarm 2 Press Switch 1 once again to activate Alarm 3 Press Switch 2 to activate the Minute Press Switch 1 repeatedly to set the desired Minute Press Switch 2 to activate the Hour Press Switch 1 repeatedly to set the desired Hour, AM or PM Press Switch 2 to set Alarm 3 ON. Checking Alarms ON/OFF: Alarms ON if in screen 2 (2 presses) Alarms lights are ON. Alarms OFF if in screen 2 (2 presses) Alarms lights are OFF Checking Alarms Setting: Press Switch 1 4 times to see Alarm 1 Setting (screen 4) Press Switch 1 once again to see Alarm 2 Setting (screen 5) Press Switch 1 once again to see Alarm 3 Setting (screen 6) Press Switch 1 once again to leave the screen. Switching Alarms OFF: Press Switch 1 4 times to go to Alarm 1 (screen 4) Press Switch 2 once to switch Alarm 1 OFF Press Switch 1 once to go to Alarm 2 (screen 5) Press Switch 2 once to switch Alarm 2 OFF Press Switch 1 once to go to Alarm 3 (screen 6) Press Switch 2 once to switch Alarm 3 OFF Press Switch 1 once to leave the screen. Note: When Alarms are OFF. Screens 4, 5, 6 are unavailable 3/ HOUR CHIME SETTING: Turn Hour Chime ON/OFF: Press & hold Switch 1 until Alarm 1 flashes then release it Press Switch 1 once again to go to Alarm 2 Press Switch 1 once again to go to Alarm 3 Press Switch 1 once again to go to Hour Chime (Screen 7) Press Switch 2 repeatedly to set Hour Chime ON or OFF Press Switch 1 2 times to leave the screen. | Checking Hour Chime: Press Switch 1 once to go to Time Reading Screen. Press Switch 1 once again to see if Hour Chime light is ON Press Switch 2 once to leave the screen. 4/ KEY TONE SETTING: Turn Key tone ON/OFF: Press & hold Switch 1 until Alarm 1 flashes Press Switch 1 once again to go to Alarm 2 Press Switch 1 once again to go to Alarm 3 Press Switch 1 once again to go to Hour Chime (Screen 7) Press Switch 1 once again to go to Key tone (Screen 8) Press Switch 2 repeatedly to set Key tone ON or OFF Press Switch 1 once to leave the screen. Checking Key tone: Press Switch 1 once to go to Time Reading Screen. Press Switch 1 once again to see if Key tone light is ON Press Switch 2 once to leave the screen. ENTER Stop mode WHEN IN SLEEP MODE Press & hold Switch 1 & Switch 2 simultaneously until you see the lights Press & hold Switch 2 until you hear a long beep Your Watch is in stop mode. Wake up from Stop mode. Press & hold Switch 1 until you hear a long beep ### Your Watch is ready to use! ENTER FLICKERING MODE: Press & hold Switch 2 then press Switch 1 Flickering Mode is ON. Repeat the same sequence, Flickering Mode is OFF BATTERY REPLACEMENT: PICTURE 1 Battery Type: CR2032 / 3V |  |
| --- | --- | --- | --- |